Mount for Laser Diode and Beam Shaping Optics

### Violet Laser Pointer Design

Final Report

Problem: To mount a plastic aspheric lens, and a pair of anamorphic prisms in front of a laser diode to create a circular beam. The components should be integrated in a single assembly. The final mount should be circular and no larger than 1 in diameter, compatible with Thorlabs SM1 threaded mounts. The circuit board in the back of the laser is 18 x 10 mm. The mount should enclose the circuit board and allow a 3 mm diameter wire bundle to come out of the back. The beam should come out of the center of the mount. The design should consider alignment stability for packing and shipping.

## Requirements:

| Optical | The beam diameter (1/e2) at output of mount = 5 mm Laser wavelength = 405 nm Laser operating power: 120 mW Laser diode used: Nichia NDV4313 Laser divergence before beam shaping optics: - Perpendicular: 19° - Parallel: 9° The laser datasheet is included below. The beam deviation coming out of the mount should be < 0.1 ° |
| --- | --- |
| Mechanical | - Circular mount, maximum mount diameter = 1.0 “ - Permanent, stable alignment - The light should come out of the center of the mount - Mount encloses the circuit board (18 x 10 mm) and wire comes out of the back - Has thread for Thorlabs SM1 mounts |
| Survival | Operating temperature: 25°C ± 5°C Survival temperature: -20°C – 70°C Shock: 20 G |

Table1. Design Requirements.

Design Preferences:

To keep the cost of the design low, use off-the-shelf optics. The following components are preferred because of their availability and low cost.

- Plastic asphere CAX046 from Thorlabs

- PS870 - Unmounted Prism Pair, Uncoated

### Design Concept

### The design of the mount for the laser diode and beam shaping optics consists of using a barrel, 25 mm in diameter and designing a mount for the anamorphic prisms, the collimating lens and the laser diode, that can be aligned outside and then mounted inside the barrel. The lens mounts and the barrel will be made out of anodized aluminum. The complete assembly is shown below in Figure 1. There are two subassemblies, the collimation and diode mount and the anamorphic prism mount.

### Figure 1. Top: view of subassemblies, Bottom: complete assembly

### The Collimation Lens

The typical angle of divergence of the laser diode is 19.5° x 9° (FWHM) . This corresponds to an NA of 0.33. to produce a 5 mm beam with this NA, the focal length  of the collimation lens should be 7.06 mm.

It is of interest for this design that off-the-shelf components are used to lower the cost. The initial recommendation was to use a plastic aspheric lens. Molded plastic aspheric lenses are cost effective to design only if the volumes are high (> 10,000). This application assumes that the volumes will not be as high, so designing custom-made collimating lenses would not be cost-effective. Having a molded glass asphere might be a better option although the unit price of these lenses is much higher than that of the plastic aspheres.

| ### Collimating Lens | ### Price | ### Focal Length – NA | ### Beam Size (mm) | ### Anamorphic Magnification Required |
| --- | --- | --- | --- | --- |
| ### Mounted Molded Glass Asphere | ### $ 119.00 | ### 8 mm – 0.50 | ### 5.41 x 2.42 | ### 2.23 |

Table 2. Collimation lens properties.

This lens has a anti-reflective coating, for 400-600 nm. The curve for this coating appears below.

Figure 2. Collimation Lens AR-Coating (Red).

The typical beam diameter would be 5.25 mm, but an aperture will set the size of the beam at 5 mm. The diameter of the collimation lens is 9.94 mm and the center thickness is 3.69 mm. Since this lens is small, it will be mounted in a cell to provide a hard surface for assembly and alignment. The lens cell has thread and will be screwed to another mount that supports the laser diode also. This mount provides the correct height for the lens and the diode since the prisms cause the beam to deviate 4.52 mm from the center   (figure 3).  There is a spring providing a preload for the lens to allow for small adjustments by increasing the friction on the threads between the collimation lens mount and the collimation lens/laser mount. There is a flat washer between the lens and the spring to prevent the spring from touching the lens directly. The details of the spring design are found in the appendix.

Spring

Laser Diode

Lens Mount

Collimation Lens

Washer

Collimation Lens/ Laser Diode Mount

Figure 3. Lens Cell Mount

The concept for mounting the collimation lens was found in the following link: http://www.kellerstudio.de/repairfaq/sam/lsrptr1.gif. This concept seems to be a common way of mounting collimation lenses for laser pointers. This is illustrated in figure 4.

Figure 4. Concept for mounting the collimation lens

## The Anamorphic Prism Pair

The prism mount consists of a barrel cut in a rectangular space to insert the prisms. Each prism swings on the mount since it is glued to one pin on each side. Figure 5 illustrates the concept taken from Yoder’s book. This is done to achieve the best alignment of the prism. Once the optimum angle is reached, the pins are secured by “liquid pinning” (Vukobratovich p. 124) as seen on Figure 6. Figure 7 shows the actual mount with the prisms. The details will be seen in the assembly and alignment section.

Figure 5. Concept for mounting the anamorphic prisms.

Figure 6. Liquid pinning concept.

23°

-54.5°

Figure 7. Top: Anamorphic Prism Pair mount, Bottom: Prisms as seen from the inside of the mount.

The prism will be adhered to the mount using Summers Milbond adhesive. According to Vukobratovich this is the best option for mounting glass to metal.

- The anamorphic prism pair requires a similar AR-coating to that of the lens.

- The prisms require angles of -54.5°, and 23° to achieve the required magnification. These angles were found using OSLO and also by calculation using the equations described in the appendix.

### The laser diode

The laser diode mounts to a mount in front of the lens. The laser diode circuit board is in the back of the diode and fits into a cut in the mount. The laser diode should be rotated to the right orientation before it is attached to the circuit board so that its small direction is in the plane that the prisms expand. There is a spring between the mounts to control the distance between the lens and the diode for collimation.

### Billing of Materials

| Item | Cost (per unit) |
| --- | --- |
| Anamorphic Prism Pair with AR-coating (400 nm -600 nm) | $ 95.00 each (for quantity of 500) |
| Molded Glass Asphere | $112.00 |
| Machining of mechanical parts | $ 500.00 |
| Laser diode | $ 1590.428 each (for quantity of 100) |
| Laser driver board | $ 30.00 |

Table 3. Billing of materials.

## Error Budget

|  | Tolerance |  |  |
| --- | --- | --- | --- |
| Lens Decenter | 13 um | Beam deviation | 0.093 deg |
| Prism 1 angle | .06 deg |  | -0.1 deg |
| Prism 2 angle | .033 deg |  | 0.097 deg |
| Lens defocus | +10 um | Beam size | 5.66109 mm |
|  | - 10 um |  | 5.71331 mm |

Nominal Beam size: x = 5.6872 y = 5.6607

## Assembly and Alignment procedure

Laser Diode Collimation Assembly

Figure 8. Diode Collimation Assembly.

- Make sure all surfaces are clean.

- The laser diode/collimation lens mount is held in place by a clamp.

- Place the spring inside the mount.

- Pick up the lens with a suction cup and place it inside the collimator lens mount.

- Place the protective washer inside the mount making sure it makes full contact with the back surface of the lens.

- Wearing an electrostatic discharge protective strap (to protect the laser diode), laser diode should be soldered to the circuit board.

- Insert the laser diode in the mount.

- Screw the lens mount on to the laser diode/collimation lens mount.

- Connect the circuit to a power supply and turn the laser on.

- Rotate the laser so that its small dimension is vertical. The light profile should look like this when projected on a screen.

- Turn the laser off and place the spacer that covers the circuit board around it until it makes contact with the diode mount.

Anamorphic Prism Assembly

Figure 9. Anamorphic Prism Assembly

- Hold the prism mount using a clamp.

- Mark the approximate location that the prisms have to be tilted to: 54.5° from the vertical for the front prism, and 47.5° for the second prism.

Hole on mount – Back prism

47.5°

-54.5°

Hole on mount – Front Prism

- Place the front prism (the one facing the light), inside the mount, with the slanted surface facing the direction of the light, making it rest on a block that provides the right height.

- Take the pin with the cut end, and place adhesive on the cut faces.

- Insert the pin with the cut end through the hole in the right side of the mount. Make sure the pin makes full contact with the prism on two surfaces: the front face and the right side.

- Take the pin with the notch and place adhesive on one of the faces at the ends. Insert the pin through the hole on the left of the mount with the adhesive facing the prism. Make sure the pin makes full contact with the prism. Allow the adhesive to dry (3 hours).

- Repeat the same procedure for the second prism.

Alignment

- Collimate the laser by letting the light shine on a distant wall (5 m away). Screw or unscrew the lens mount until the size of the beam is constant at different points in the path. Insert a card at different points on the path of the beam to see this.

- Alignment setup: place the collimation assembly in front of two small apertures, 2 mm in diameter mounted on an optical rail. One of the apertures should be close to the laser and another one far away (3 m).

- Align the lens by inserting a pointy object inside the holes in the mount to push on the lens. This will adjust the centering of the lens controlling the deviation of the beam. The closest aperture controls the x-y motion; the second aperture controls the beam deviation (see figure 10 below).

Figure 10. Alignment of the beam using the apertures.

- Insert the rods in the collimation mount and the prism mount on the other side.

- Rotate the front prism to the location marked.

- Fill the notch in the left pin with adhesive.

- Do the same with the second prism (the one at a lower height in the mount)

- Do the final adjustment using the two small apertures of control the beam divergence.

- Use Newport’s beam profiler to measure the beam size and make the fine adjustments.

- Allow the adhesive to dry (3 hours)

- Insert the spacer into the barrel and make sure it makes full contact with the back of the barrel.

- Insert the collimation assembly together with the prism assembly. The circuit board for the laser should fit into the aperture of the spacer, and the back face of the diode mount should make full contact with the front face of the spacer.

- Take the threaded cap and cover the threads with adhesive. Screw it on to the entrance of the barrel. It should hold all the components in place.

Temperature

The assembly stands the temperature range  from -20°-70° C. The machined parts are all made out of aluminum so there is not CTE mismatch. The lens only requires a clearance of 4 μm to avoid touching the mount with thermal expansion.  The adhesive bond between the prism and the mount causes a shear stress of 293 Pa between the two surfaces, which is very small.

Shock

The assembly can resist 20 G’s of shock. The cap of the barrel needs to provide a preload of 9.65 N to make the assembly safe for this acceleration. The lens requires a preload of 0.15 N, and the spring needs a force of 5.13 N to compress to the ideal length, so that would be the preload on the lens. The prism requires a preload of 0.37 N. The stiffness of the adhesive is enough to support the prism weight if the bond is at leas 100 μm2 . The actual area of the bond is 6.3 mm2 .

## Conclusion

A laser diode at 405 nm was mounted meeting all the requirements. The details of the design are found in the assembly.

## References:

Spring Design Equations: http://www.mech.uwa.edu.au/DANotes/springs/intro/intro.html#top

Anamorphic Prisms Calculations: Svelto, Orazio. Principles of Lasers. 4th ED. Springer.  P.215-217

Vukobratovich, Daniel. Notes from Fall 2007.

Yoder Jr., Paul R. Opto-Mechanical Systems Design. 3rd ED. Taylor and Francis.

### Appendix

Optical System Model in OSLO

Figure 11. Optical System Model

Figure 12. Beam size after the collimation lens.

Figure 13. Beam size at 20 mm after the exit of the barrel mount.

Figure 14. Optical model parameters

|  |  |  | Aluminum | SF11 | S-LAL13 | Summers Milbond Adhesive |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Property |  |  | 6061-T6 |  |  |  |  |
| Refractive Index at 405 nm |  |  |  | 1.84235 | 1.71566 |  |  |
| CTE α (x10-6/°C) |  |  | 23.6 | 8.5 | 5.7 | 62 | at -54 to 20 |
|  |  |  |  |  |  | 72 | at 20 to 70 |
| Young's Modulus E (x1010 Pa) |  |  | 6.82 | 9.2 | 10.73 | 0.0158 | at 20 °C |
| Poisson's Ratio ν |  |  | 0.332 | 0.257 | 0.29 | 0.43 |  |
| Density ρ (g/cm3) |  |  | 2.68 | 3.22 | 3.6 |  |  |
| Specific Heat Cp (J/kg K) |  |  | 960 | 710 |  |  |  |
| Thermal Conductivity (W/m K) |  |  | 167 | 0.95 | 0.893 |  |  |
| Shear strength (Mpa) |  |  |  |  |  | 14.5 | at 25 °C |
|  |  |  |  |  |  | 6.8 | at 70 °C |
| Joint Thickness (mm) |  |  |  |  |  | 0.381 |  |
| Temperature range (°C) |  |  |  |  |  | (-54 to 70) |  |
| Curing time (hours) |  |  |  |  |  | 3 | at 71 °C |

Table 4. Material Properties.

## Anamorphic Prism Angles

The angles of tilt of the anamorphic prisms were found using OSLO and confirmed using the equations found on Principles of Lasers (p. 215 – 217). This analysis found the ideal angles to be -52.93° for the first surface of the first prism and 25.66° for the second prism. The actual angle (as calculated by OSLO) of the first prism is -54.5°, and 23° for the second prism.  This produces a beam diameter of 5.68 mm. The diameter of the beam is clipped to 5 mm by using an aperture at the exit of the mount.

The beam sizes in the perpendicular and parallel directions of the diode are

D⊥=2ftanθ⊥

D∥=2ftanθ∥

The required magnification is found using the ratio of the beam size in the perpendicular and parallel directions of the diode

M=D⊥D∥12

This is an iterative procedure that involves starting with an arbitrary angle of incidence and using it in Snell’s law to find a first value for the angle of refraction.   The magnification of each prism is given by

M=DrDi=cosθrcosθi

θ2=sin-1n1sinθ1n2

Beam divergence in parallel direction in degrees

theta_par =  9

Beam divergence in perpendicular direction in degrees

theta_per =  19.5000

Focal length of collimation lens in mm

f = 8

Beam size in the parallel direction in mm

d_par =  2.5342

Beam size in the perpendicular direction in mm

d_per = 5.6659

Required Anamorphic magnification

M = 1.4953

Angle of the first prism in degrees

theta1_final = 52.9287

Angle of the second prism in degrees

theta2_final = 25.6630

% snell.m --- Calculates the angles of tilt a set of anamorphic prisms.

clear all

clc

deg2rad = pi/180; % Degree to radian conversion

fprintf('Beam divergence in parallel direction in degrees')

theta_par = 9

fprintf('Beam divergence in perpendicular direction in degrees')

theta_per = 19.5

fprintf('Focal length of collimation lens in mm')

f = 8

fprintf('Beam size in the parallel direction in mm')

d_par = 2*f*tan(theta_par*deg2rad)

fprintf('Beam size in the perpendicular direction in mm')

d_per = 2*f*tan(theta_per*deg2rad)

fprintf('Required Anamorphic magnification')

M = sqrt(d_per/d_par)

theta1 = 50; % Starting angle to enter the iterative process

n2 = 1.84235; % Index of refraction of the prism material

n1 =1; % Index of refraction in air

x = 1; % Counter - Test value start the loop

theta1 = theta1*deg2rad; % Conversion of the starting angle from degrees to radians

% Iterative process to find the angle of the prisms

while abs(x) > 0.0001

theta2 = asin((n1*sin(theta1))/n2);

theta_i = acos(cos(theta2)/M);

x = theta_i - theta1;

theta1 = theta_i;

end

fprintf('Angle of the first prism in degrees')

theta1_final = theta1/deg2rad

fprintf('Angle of the second prism in degrees')

theta2_final = theta2/deg2rad

## Spring Design

The spring will be made out of carbon steel and has dimensions of 11 mm in diameter and a length of 8 mm. The pitch is 2 mm and the wire diameter is 1 mm. The spring constant of this spring is 3.29 N/mm. Ideally the distance from the lens to the diode is 6.44 mm. The spring has to be compressed by a distance of 1.56 mm. This requires a force of 5.13 N.

clear all

clc

d = 1;

D0 = 11;

D = D0 - d; % Mean coil diameter

C = D/d; % Index

Ks = 1+0.5/C; % Stress factor (static)

Kh = (C+0.6)/(C-0.67); % Stress Factor (fatigue)

nt = 3; % Total number of turns

na = nt-2 % Number of active turns

Ls = nt*d % Solid length

p = 2; % pitch in mm

L0 = p*na+2*d % Length of spring undisturbed

alpha = atan(p/D)*180/pi % Helix angle

G = 79e9; % Modulus of rigidity for carbon steel in Pa

k = G*(d/1000)/(8*na*C^3) % Spring constant

delta_s = L0-Ls % Solid deflection

Fs = k*delta_s % Solidification load

tau_s = Ks*8*Fs*C/(pi*d^2) % Solid shear stress

delta_hi = delta_s/1.1 % For 10% clash allowance

F_hi = k*delta_hi;

tau_hi = Ks*8*F_hi*C/(pi*d^2)

lambda = 1 % Constant assuming hinged ends

c2 = 2.62

b = lambda*L0/(c2*D) % If >1, buckling will occur

dist = L0 - 6.44 % COmpression distance required to achieve correct distance from lens to laser

F = k*dist % Force required to compress the spring to the correct distance

Result:

na = 3

Ls = 5

L0 = 8

alpha = 11.3099

k = 3.2917

delta_s = 3

Fs = 9.8750

tau_s = 264.0381

delta_hi = 2.7273

tau_hi = 240.0346

lambda = 1

c2 = 2.6200

b = 0.3053

dist = 1.5600

F = 5.1350